Laboratory #8

Second-Order Circuits

- Objectives

  - Understand the characteristics and applications of second-order circuits.

  - Understand the step response of RLC circuits.

- Components and Instruments

  - Components

    - Fixed resistors: 47 kΩ × 1 and 1 kΩ × 2

    - Fixed capacitors: 100 μF × 1, 0.01 μF × 1, and 0.1 μF × 1

    - Fixed inductors: 100 mH × 1 and 470 mH × 1

  - Instruments

    - DC power supply

    - Function generator

    - Digital multimeter

    - Oscilloscope

- Reading

Concentration will be on Chapter 8 of the Textbook, C. K. Alexander and M. N. O. Sadiku, Fundamentals of Electric Circuits, 6th edition, New York, NY, USA: McGraw-Hill, 2016.

- Preparation

  - Source-free series RLC circuits

An understanding of the natural response of the series RLC circuit is a necessary background for future studies in filter design and communications networks.

Consider the series RLC circuit shown in Fig. 8.1. The circuit is excited by the energy initially stored in the capacitor and the inductor.

- A source-free series RLC circuit.

The energy is represented by the initial capacitor voltage V0 and the initial inductor current I0. Thus, the second-order differential equation of Fig. 8.1 is given by

|  | (8-1) |
| --- | --- |

Substitute  into (8-1), the following equation can be obtained

|  | (8-2) |
| --- | --- |

Since  is the assumed solution to be found, only the expression in parentheses in (8-2) can be zero:

|  | (8-3) |
| --- | --- |

This quadratic equation is known as the characteristic equation. Since the roots of the equation dictate the characteristics of i, the two roots are:

|  | (8-4) |
| --- | --- |
|  | (8-5) |

A more compact form to express the two roots is

|  | (8-6) |
| --- | --- |
|  | (8-7) |

where

|  | (8-8) |
| --- | --- |

The roots s1 and s2 are called natural frequencies, measured in nepers per second (Np/s), because they are associated with the natural response of the circuit. The ω0 is known as the resonant frequency or strictly as the undamped natural frequency, expressed in radians per second (rad/s). The α is the neper frequency or the damping factor, expressed in nepers per second. In terms of α and ω0, equation (8-3) can be rewritten as

|  | (8-9) |
| --- | --- |

Thus, the natural response of the series RLC circuit is

|  | (8-10) |
| --- | --- |

From equations (8-6) and (8-7), we can infer that there are three types of solutions

- If α > ω0, it is the overdamped case.

From equations (8-6) and (8-7), α > ω0 implies C > 4L/R2. When this happens, both roots s1 and s2 are negative and real. The response is

|  | (8-11) |
| --- | --- |

which decays and approaches zero as t increases. Fig. 8.2 illustrates a typical overdamped response.

- Overdamped response.

- If α = ω0, it is the critically damped case.

When α = ω0, C = 4L/R2 and

|  | (8-12) |
| --- | --- |

For this case, equation (8-10) yields

|  | (8-13) |
| --- | --- |

Hence, the natural response of the critically damped circuit is

|  | (8-14) |
| --- | --- |

A typical critically damped response is shown in Fig. 8.3. It is a sketch of which reaches a maximum value of , which reaches a maximum value of e-1/α at t = 1 s, one time constant, and then decays all the way to zero.

- Critically damped response.

- If α < ω0, it is the underdamped case.

For α < ω0 and C = 4L/R2, the roots may be written as

|  | (8-15) |
| --- | --- |
|  | (8-16) |

where  and  which is called the damping frequency or the damped natural frequency. Both ω0 and ωd are natural frequencies because they help determine the natural response. The ω0 is often called the undamped natural frequency while ωd is called the damped natural frequency. The natural response is

|  | (8-17) |
| --- | --- |

Using Euler’s identities, we get

|  | (8-18) |
| --- | --- |

Replacing constants (A1 + A2) and j(A1 - A2) with constants B1 and B2, equation (8-18) can be rewritten as

|  | (8-19) |
| --- | --- |

With the presence of sine and cosine functions, it is clear that the natural response for this case is exponentially damped and oscillatory in nature. The response has a time constant of 1/α and a period of 2/ωd. Fig. 8.4 depicts a typical underdamped response.

- Underdamped response.

  - Step response of a series RLC circuit

As we learned in the preceding section, the step response is obtained by the sudden application of a DC source to a series RLC circuit. Consider the series RLC circuit shown in Fig. 8.5 and apply KVL around the loop for t > 0 s.

- Step voltage applied to a series RLC circuit.

Thus, we get the second-order differential equation as below.

|  | (8-20) |
| --- | --- |

The solution to equation (8-20) has two components: the transient response vt(t) and the steady-state response vss(t), that is,

|  | (8-21) |
| --- | --- |

The transient response vt(t) is the component of the total response that dies out with time. The form of the transient response is the same as the form of the solution obtained in Section 1. For the source-free series RLC circuit, it is given by equations (8-11), (8-14), and (8-19). Therefore, the transient response vt(t) for the overdamped, underdamped, and critically damped cases are, respectively,

|  | (8-22) |
| --- | --- |
|  | (8-23) |
|  | (8-24) |

The steady-state response is the final value of v(t). In the circuit in Fig. 8.5, the final value of the capacitor voltage is the same as the source voltage vs. Hence,

|  | (8-25) |
| --- | --- |

Thus, the complete solutions for the overdamped, underdamped, and critically damped cases are, respectively,

|  | (8-26) |
| --- | --- |
|  | (8-27) |
|  | (8-28) |

The values of the constants A1 and A2 are obtained from the initial conditions: v(0) and dv(0)/dt. Alternatively, the complete response for any variable can be found directly, because it has the general form

|  | (8-29) |
| --- | --- |

where xss = x(∞) the is the final value and xt(t) is the transient response. The transient response has the same form as in equations (8-22)-(8-24), and the associated constants are determined from equations (8-26)-(8-28) based on the values of x(0) and dx(0)/dt.

  - General second-order circuits

Now we are prepared to apply the ideas to any second-order circuit having one or more independent sources with constant values. Given a second-order circuit, we determine its step response x(t) (which may be voltage or current) by taking the following four steps:

- We first determine the initial conditions x(0), dx(0)/dt, and the final value x(∞).

- We turn off all the independent sources and find the form of the transient response xt(t) by applying KCL and KVL. Once a second-order differential equation is obtained, we determine its characteristic roots. Depending on whether the response is overdamped, critically damped, or underdamped, we obtain xt(t) with two unknown constants as we did in the previous section.

- We obtain the steady-state response as

|  | (8-30) |
| --- | --- |

where x(∞) is the final value of x, obtained in step (1).

- The total response is now found as the sum of the transient response and steady-state response

|  | (8-31) |
| --- | --- |

We finally determine the constants associated with the transient response by imposing the initial conditions x(0) and dx(0)/dt determined in step (1).

We can apply this general procedure to find the step response of any second-order circuit, including those with op amps. The following examples illustrate the four steps.

- Explorations

  - A series RLC charging circuit

    - Connection diagram is as follows:

- A series RLC charging circuit.

    - Record:

Use the power supply to provide a DC signal of 10 V for the circuit. When the switch “SW1” turns on, using the digital multimeter to measure the voltage of vR and vC, and then record the voltage data. The voltage data must be recorded every 4 seconds for a total of 40 seconds. Note that the passive components have to completely discharge before this experiment.

| t (s) | vR (V) | vC (V) | Theoretical value of vR (V) | Theoretical value of vC (V) |
| --- | --- | --- | --- | --- |
| 0 |  |  |  |  |
| 4 |  |  |  |  |
| 8 |  |  |  |  |
| 12 |  |  |  |  |
| 16 |  |  |  |  |
| 20 |  |  |  |  |
| 24 |  |  |  |  |
| 28 |  |  |  |  |
| 32 |  |  |  |  |
| 36 |  |  |  |  |
| 40 |  |  |  |  |

    - Calculate iR and the energy stored in the capacitor C using the above measured values:

| t (s) | iR = (A) | EC = (J) |
| --- | --- | --- |
| 0 |  |  |
| 4 |  |  |
| 8 |  |  |
| 12 |  |  |
| 16 |  |  |
| 20 |  |  |
| 24 |  |  |
| 28 |  |  |
| 32 |  |  |
| 36 |  |  |
| 40 |  |  |

    - Draw the charging curves of vR and vC in the figures as follows.

| Waveform of vR | Waveform of vC |
| --- | --- |

  - A series RLC discharging circuit

    - Connect the circuit as shown in Fig. 8.6.

    - Record:

When the switch “SW1” is still closed and the DC voltage source of 10 V is suddenly set to be 0 V, using the digital multimeter to measure the voltages of vR and vC and record the voltage data. The data must be recorded every 4 seconds for a total of 40 seconds. Note that the passive components have to be fully charged before the experiment.

| t (s) | vR (V) | vC (V) | Theoretical value of vR (V) | Theoretical value of vC (V) |
| --- | --- | --- | --- | --- |
| 0 |  |  |  |  |
| 4 |  |  |  |  |
| 8 |  |  |  |  |
| 12 |  |  |  |  |
| 16 |  |  |  |  |
| 20 |  |  |  |  |
| 24 |  |  |  |  |
| 28 |  |  |  |  |
| 32 |  |  |  |  |
| 36 |  |  |  |  |
| 40 |  |  |  |  |

    - Calculate iR and the energy stored in the capacitor C using the above measured values.

| t (s) | iR = (A) | EC = (J) |
| --- | --- | --- |
| 0 |  |  |
| 4 |  |  |
| 8 |  |  |
| 12 |  |  |
| 16 |  |  |
| 20 |  |  |
| 24 |  |  |
| 28 |  |  |
| 32 |  |  |
| 36 |  |  |
| 40 |  |  |

    - Draw the discharging curves of vR and vC in the figures as follows.

| Waveform of vR | Waveform of vC |
| --- | --- |

  - A second-order RLL circuit

    - Connection diagram is shown in Fig. 8.7.

Note that the input voltage of VP-P is a square wave with a peak-to-peak value of 10 V and a frequency of 1 kHz.

- A second-order RLL circuit.

    - Use the oscilloscope to measure the voltage waveforms across R1, L1 and L2.

      - In CH1, the positive terminal is connected to point A and the negative terminal is connected to point D to measure the voltage waveform of VP-P.

      - In CH2, the positive terminal is connected to point A and the negative terminal is connected to point B to measure the voltage waveform of R1.

      - In CH3, the positive terminal is connected to point B and the negative terminal is connected to point C to measure the voltage waveform of L1.

      - In CH4, the positive terminal is connected to point C and the negative terminal is connected to point D to measure the voltage waveform of L2//R2.

      - Setting of measured values: Press Measure button first, then press the first button on the right-hand side of the screen to enter the setting of the measurement values. The first and second sources are selected as CH1, CH2, CH3, and CH4, respectively.

      - Press Save to save the voltage waveforms in USB.

  - A second-order RCC circuit

    - Connection diagram is shown in Fig. 8.8.

Note that the input voltage of VP-P is a square wave with a peak-to-peak value of 10 V and a frequency of 1 kHz.

- Second-order RCC Circuit.

    - Use the oscilloscope to measure the voltage waveforms across R1 and R2.

      - In CH1, the positive terminal is connected to point A and the negative terminal is connected to point D to measure the voltage waveform of VP-P.

      - In CH2, the positive terminal is connected to point A and the negative terminal is connected to point B to measure the voltage waveform across R1.

      - In CH3, the positive terminal is connected to point B and the negative terminal is connected to point C to measure the voltage waveform across R2.

      - Setting of measured values: Press Measure button first, then press the first button on the right-hand side of the screen to enter the setting of the measurement values. The first and second sources are selected as CH1, CH2, and CH3, respectively.

      - Press Save to save the voltage waveforms in USB.

    - Use the oscilloscope to measure the voltage waveforms across C1 and C2.

      - In CH1, the positive terminal is connected to point A and the negative terminal is connected to point D to measure the voltage waveform of VP-P.

      - In CH2, the positive terminal is connected to point B and the negative terminal is connected to point D to measure the voltage waveform across C1.

      - In CH3, the positive terminal is connected to point C and the negative terminal is connected to point D to measure the voltage waveform across C2.

      - Setting of measured values: Press Measure button first, then press the first button on the right-hand side of the screen to enter the setting of the measurement values. The first and second sources are selected as CH1, CH2, and CH3, respectively.

      - Press Save to save the voltage waveforms in USB.

Laboratory #8 Pre-lab

Class:

Name:                         Student ID:

Note that in these simulations, the input voltage of VP-P is a square wave with a peak-to-peak value of 10 V and a frequency of 1 kHz.

- Connect the circuit as shown in Fig. 8.6 and simulate the circuit using PSpice. Show the voltage waveforms which are across R and L, respectively. Please attach the schematic and simulation results.

- Connect the circuit as shown in Fig. 8.7 and simulate the circuit using PSpice. Show the voltage waveforms which are across R1, L1, and L2, respectively. Please attach the schematic and simulation results.

- Connect the circuit as shown in Fig. 8.8 and simulate the circuit using PSpice. Show the voltage waveforms which are across R1, R2, C1, and C2, respectively. Please attach the schematic and simulation results.

- Connect the circuit as follow.

- RLC circuit in parallel.

Simulate the circuit by using Matlab. Consider three different cases: R = 1.923 Ω, R = 5 Ω, and R = 6.25 Ω. Show the voltage waveform of v(t) for t = 0-10 s by assuming v(0) = 5 V, i(0) = 0, L = 1 H, and C = 10 mF. Please attach simulation results.

Laboratory #8 Report

Class:

Name:                         Student ID:

- Exploration 1

- Record

| t (s) | vR (V) | vC (V) | Theoretical value of vR (V) | Theoretical value of vC (V) |
| --- | --- | --- | --- | --- |
| 0 |  |  |  |  |
| 4 |  |  |  |  |
| 8 |  |  |  |  |
| 12 |  |  |  |  |
| 16 |  |  |  |  |
| 20 |  |  |  |  |
| 24 |  |  |  |  |
| 28 |  |  |  |  |
| 32 |  |  |  |  |
| 36 |  |  |  |  |
| 40 |  |  |  |  |

- Calculate iR and the energy stored in the capacitor C using the above measured values:

| t (s) | iR = (A) | EC = (J) |
| --- | --- | --- |
| 0 |  |  |
| 4 |  |  |
| 8 |  |  |
| 12 |  |  |
| 16 |  |  |
| 20 |  |  |
| 24 |  |  |
| 28 |  |  |
| 32 |  |  |
| 36 |  |  |
| 40 |  |  |

- Draw the charging curves of vR and vC in the figure as follows.

| Waveform of vR | Waveform of vC |
| --- | --- |

- Exploration 2

- Record

| t (s) | vR (V) | vC (V) | Theoretical value of vR (V) | Theoretical value of vC (V) |
| --- | --- | --- | --- | --- |
| 0 |  |  |  |  |
| 4 |  |  |  |  |
| 8 |  |  |  |  |
| 12 |  |  |  |  |
| 16 |  |  |  |  |
| 20 |  |  |  |  |
| 24 |  |  |  |  |
| 28 |  |  |  |  |
| 32 |  |  |  |  |
| 36 |  |  |  |  |
| 40 |  |  |  |  |

- Calculate iR and the energy stored in the capacitor C using the above measured values.

| t (s) | iR = (A) | EC = (J) |
| --- | --- | --- |
| 0 |  |  |
| 4 |  |  |
| 8 |  |  |
| 12 |  |  |
| 16 |  |  |
| 20 |  |  |
| 24 |  |  |
| 28 |  |  |
| 32 |  |  |
| 36 |  |  |
| 40 |  |  |

- Draw the charging curves of vR and vC in the figure as follows.

| Waveform of vR | Waveform of vC |
| --- | --- |

- Exploration 3

- Use the oscilloscope to measure the voltage waveforms across R1, L1 and L2.

- Exploration 4

- Use the oscilloscope to measure the voltage waveforms across R1 and R2.

- Use the oscilloscope to measure the voltage waveforms across C1 and C2.

- Problem 1

Please explain which type of damping is in the circuit shown in Fig. 8.6, and explain the reason?

- Problem 2

Please design the circuit in Fig. 8.6 for two other damping conditions, and list the values of resistance under different damping, separately. (If the answer of problem 1 is critically damping, then design the circuit as underdamping and overdamping in this problem). Note that the values of the inductor L and the capacitor C are fixed and only the value of resistor can be changed.

- Problem 3

In the second-order op amp circuit as follow. Simulate the circuit by using Matlab. Show the voltage waveform of v(t) for t = 0-10 s. Please attach simulation results.

- Circuit diagram.

- Conclusion